MODEL: S600

1. Safety Instructions

Please read the instruction carefully which includes important information

about the installation, usage and maintenance.

l Please keep this User Guide for future consultation. If you sell the unit to another user, be sure that they also receive this instruction manual.

l Unpack and check carefully to ensure that there is no transportation damage before using the unit.

l This product is suitable for wet locations. Do not immerse in water.

l DO install and operate by qualified operator.

l DO NOT allow children to operate the fixture.

l Use safety chain when fixing the unit. Handle the unit by carrying its base instead of head only.

l The unit must be installed in a location with adequate ventilation, at least 50cm from adjacent surfaces.

l Be sure that no ventilation slots is blocked, otherwise the unit will be overheated.

l Before operation, ensure that you are connecting this product to the proper voltage in accordance with the specifications in this manual or on the

product’s specification label.

l It’s important to ground the yellow/green conductor to earth in order to avoid electric shock.

l Minimum ambient temperature TA: -40℃ . Maximum ambient temperature TA: 160℃ . Do not operate this product at a lower or higher temperature.

l DO NOT connect the device to any dimmer pack

l Keep flammable materials away from the fixture while operating to avoid fire hazard.

l Make sure the power cord is not crimped or damaged; replace it immediately if damaged

l Unit’s surface temperature may reach up to 65℃ . DO NOT touch the housing bare-handed during its operation

l Avoid any flammable liquids, water or metal from entering the unit.

l DO NOT operate in a dirty or dusty environment. DO clean the fixture regularly.

l DO NOT touch any wire during operation as there might be a hazard of electric shock.

l Avoid entanglement of the power cord with other wires.

l The minimum distance to objects/surface must be more than 0.5 meters.

l In the event of serious operating problem, stop using the unit immediately.

l Never turn on and off the unit time after time.

l The housing, the lenses, or the ultraviolet filter must be replaced if they are visibly damaged

l DO NOT open the housing as there are no user serviceable parts inside.

l DO NOT attempt to operate this unit if it becomes damaged. DO NOT

attempt any repairs yourself. Repairs carried out by unskilled people can lead to damage or malfunction. Please contact the nearest authorized

technical assistance center if needed.

l Disconnect this product from its power source before servicing.

l DO use the original packaging if the device is to be transported.

l Avoid direct eye exposure to the light source while the product is on.

l DO NOT operate this product if you see damage on the housing, shields, or cables. Have the damaged parts replaced by an authorized technician at once.

2.INSTALLATIONS

l installations Note: In order to increase protection, please install the lamp on the sidewalk, outside the seating area, or an area where unauthorized persons may touch the led

l Before installing the fixture on any surface, make sure that the installation area can bear the minimum point load above 10 points of the weight of the equipment. The installation of the fixing device must always be fixed with auxiliary safety accessories (such as a suitable safety rope) Do not stand directly under the equipment when installing, removing, or servicing

fixtures

l Ensure thatthis fixture is kept at least 0.5m (1.5 feet) away from any flammable materials(decorations, etc.)

l Be sure to use and install the supplied safety rope to ensure safety and

prevent accidental damage and/or injury in case the fixture is damaged

Installation point: Overhead installation requires a wealth of experience,   including calculation of working load limits, in-depth understanding of the  installation materials used, and regular safety inspections of all installation materials and fixtures. If you do not have these qualifications, please do  not try to install it yourself. Improper installation can cause personal injury Before connecting the main power cord to an appropriate wall outlet,make sure to complete all assembly and installation procedures

3. DMX-512 control connections

Connect the provided XLR cable to the female 3-pin XLR output of your

controller and the other side to the male 3-pin XLR input of the moving head. You can chain multiple Moving heads be connected together through serial   linking. The cable needed should be two core, screened cable with XLR input and output connectors.

For installations where the DMX cable has to run a long distance or is in an

electrically noisy environment, such as in a discotheque, it is recommended to use a DMX terminator. This helps in preventing corruption of the digital control

signal by electrical noise. The DMX terminator is simply an XLR plug with a

120 Ω resistor connected between pins 2 and 3,which is then plugged into the output XLR socket of the last fixture in the chain. Please see illustrations below

4.Address code setting

If you use a universal DMX controller to control the units, you have to set DMX address from 1 to 512 so that the units can receive DMX signal.

Press the MENU button to enter menu mode, select DMX Settings, press the ENTER button to confirm, use the UP/DOWN button to select DMX Address, press the ENTER button to confirm, the present address will blink in the display, use the UP/DOWN button to adjust the address from 001 to 512, press the ENTER button to store. Press the MENU button back to the last menu or let the unit idle 30 seconds to exit menu mode.Please refer to the following diagram to address your DMX512 channel for the first 4 units.

| Channel MODE | UNIT1 ADDERSS | UNIT2 ADDERSS | UNIT3 ADDERSS | UNIT4 ADDERSS |
| --- | --- | --- | --- | --- |
| 22CH | 1.1 | 1.21 | 1.41 | 1.61 |
| 30CH | 1.1 | 1.37 | 1.73 | 1.109 |
| 31CH | 1.1 | 1.58 | 1.115 | 1.172 |
| 39CH | 1.1 | 1.67 | 1.133 | 1.199 |
| 40CH | 1.1 | 1.94 | 1.187 | 1.280 |

- Fixture size

- 6.Illuminance chart:

- Effect wheels

- cutting disk                                CMY+CTO

- 4Prism                                    row Prism

- Frost                                         Iris

- COLOR WHEEL                            Animation GOBO

- Rotation GOBO WHEEL

- Rotation gobos

- STATIC GOBO WHEEL

- STATIC GOBO

- Lamp technical parameters

- Input Voltage: 100-240V, 50/60Hz

- Power consumption: 1050W

- Power supply: 1200W

- Light source: 1000W Module LED Engine

- Lifetime:≥20,000Hours

- Color temperature: 7500K

- Linear CTO color temperature adjustment (3000K-7000K)

- Standard color rendering index Ra ≥ 70

- High color rendering index Ra ≥ 90

- Beam angle: 5°-50°

- Linear CMY color mixing system

- Color wheel: 1 color wheel, 6 fixed colors+CRI color that can be rotated in both directions

- Fixed Gobo : 1 fixed gobo ,9 fixed gobo

- Rotation Gobo: 1 rotating gobo , 7 Rotation Gobo, patterns can be replaced

- Animation gobo, imitating water patterns and flame effects, dynamic disc rotatable

- Prism:4-prism,4-sided straight mirror, both capable of bi-directional rotation

- Two independent frost tablets (Mild and moderate) can be used in combination

- Cutting system: 4 grating to achieve fast and smooth cutting, each grating cutting direction and Angle can be independently controlled; The entire cutting module can be rotated 120°

- 0-100% 16bit smooth dimming(Four dimming modes)

- Adjustable refresh frequency, HD camera shooting without flicker

- More strobe effects, variable speed

- DMX Channel: 22/30/31/39/40CH

- Operate mode: DMX512, master/slave

- Electronic autofocus

- Outstanding strobe effect with variable speed

- Control panel: LCD touch screen

- Intelligent protection of overheat temperature

- X-axis rotation angle: 540°

- Y-axis rotation angle: 270°

- P/T: 16 bit precision scanning, no deviation

- XY axis magnetic coding positioning is more accurate

- Three phase motor with SY Brand

- Signal socket: standard 3-pin XLR and 5-pin XLR socket or RJ45

- Standard with 50MM folding fixture

- Fixture size: 442x308x730mm

Protection level: IP65

Net weight: 38.KG

Feature：

fixture set light beam, spot, wash and cutting functions in one, pure color, uniform spot, color plate, glass gobo plate, CTO, CRI plate are schott materiallt, , Plug in CMY color system, Provides rich color saturation and clear seamless color transitions. LED adopts energy-saving and environment-friendly 1000W monochrome LED light engine, which can produce extremely high light output white light, color temperature of 7500K, CRIRA≥90, and zoom range 6°- 50,The cutting system consists of 4 gratings, which can be rotated ± 60°, giving designers unlimited spot creativity. Two high brightness atomizing pieces of new materials can be respectively inserted and superimposed,All day use, waterproof rating IP65

- Menu Description

- Attention: screen unlock password: Up Down Up Down

| First level menu | Secondary menu | Level 3 Menu | Fourth level menu |
| --- | --- | --- | --- |
| DMX set up | DMX address | 1 - 512 |  |
|  | Channel mode | 39CH |  |
|  |  | 40CH |  |
|  |  | 31CH |  |
|  |  | 30CH |  |
|  |  | 22CH |  |
|  | Run mode | DMX Auto |  |
| Personality | Pan Inverse | no |  |
|  |  | yes |  |
|  | Tilt Inverse | no |  |
|  |  | yes |  |
|  | Tilt Feed back | off |  |
|  |  | on |  |
|  | Dimming curve | Straight line |  |
|  |  | Square law |  |
|  |  | Inverse square law |  |
|  |  | s-curve |  |
|  | Dimming frequency | 1200Hz |  |
|  |  | 2400Hz |  |
|  |  | 4000Hz |  |
|  |  | 6000Hz |  |
|  |  | 25000Hz |  |
|  | Dimming speed | fast |  |
|  |  | slow |  |
|  | No-signal mode | keep |  |
|  |  | Black field |  |
|  | Fan mode | Standard |  |
|  |  | Quiet |  |
|  |  | Theater |  |
|  | Display setting | Display Rotate | normal |
|  |  |  | Rotate 180° |
|  |  | Display Intensity | 1-100 |
|  |  | Temperature unit | °C |
|  |  |  | °F |
|  |  | Screen Lock | Off/on |
|  |  | language | English |
|  |  |  | Chinese |
|  | power switch | no |  |
|  |  | yes |  |
| Motor reset | Pan/Tilt | no |  |
|  |  | yes |  |
|  | Effect | no |  |
|  |  | yes |  |
|  | ALL | no |  |
|  |  | yes |  |
| DMX Live | Channel list | 0 - 255 |  |
| manual control | Channel list | 0 - 255 |  |
| service | password | 068 |  |
| Information | Power on time | xxxxxH |  |
|  | LED hours | xxxxxH |  |
|  | SW Version | 1U：Vxx |  |
|  |  | 2U：Vxx |  |
|  |  | 3U：Vxx |  |
|  |  | 4U：Vxx |  |
|  |  | 5U：Vxx |  |
|  |  | 6U：Vxx |  |
|  |  | 7U：Vxx |  |
|  | RDM UID | 0xXXXX-XXXXXXXX |  |
|  | temperature | LED | xxx |
|  | Fan speed | Fan1 | xxx RPM |
|  |  | Fan2 | xxx RPM |
|  |  | Fan3 | xxx RPM |
|  |  | Fan4 | xxx RPM |

- Channel

- 40 Channels

| CH | Function | Value | Effect |
| --- | --- | --- | --- |
| 1 | Pan | 0-255 | Pan 0-540° |
| 2 | Pan Fine | 0-255 | Pan Fine |
| 3 | Tilt | 0-255 | Tilt 0-270° |
| 4 | Tilt Fine | 0-255 | Tilt Fine |
| 5 | P/T Speed | 0-255 | Fast to slow |
| 6 | C | 0-255 | 0-100% |
| 7 | M | 0-255 | 0-100% |
| 8 | Y | 0-255 | 0-100% |
| 9 | CTO | 0-255 | 0-100% |
| 10 | Color | 0-7 | Open |
|  |  | 8-18 | color1 |
|  |  | 19-29 | color2 |
|  |  | 30-40 | color3 |
|  |  | 41-51 | color4 |
|  |  | 52-63 | color5 |
|  |  | 64-67 | Open 1 |
|  |  | 68-71 | Open+Color 1 |
|  |  | 72-75 | Color 1 |
|  |  | 76-79 | Color 1+Color 2 |
|  |  | 80-83 | Color 2 |
|  |  | 84-87 | Color 2+Color 3 |
|  |  | 88-91 | Color 3 |
|  |  | 92-95 | Color 3+Color 4 |
|  |  | 96-99 | Color 4 |
|  |  | 100-103 | Color 4+Color 5 |
|  |  | 104-107 | Color 5 |
|  |  | 108-111 | Color 5+CRI |
|  |  | 112-115 | CRI |
|  |  | 116-119 | CRI+Open |
|  |  | 120-127 | Open 2 |
|  |  | 128-189 | Reverse rotation, from fast to slow |
|  |  | 190-193 | Stop |
|  |  | 194-255 | Rotate forward, going from fast to slow |
| 11 | Rotation Gobo | 0-7 | Open |
|  |  | 8-15 | Gobo1 |
|  |  | 16-23 | Gobo2 |
|  |  | 24-31 | Gobo3 |
|  |  | 32-39 | Gobo4 |
|  |  | 40-47 | Gobo5 |
|  |  | 48-55 | Gobo6 |
|  |  | 56-63 | Gobo7 |
|  |  | 64-72 | Gobo1shake slow to fast |
|  |  | 73-81 | Gobo2shake slow to fast |
|  |  | 82-90 | Gobo3shake slow to fast |
|  |  | 91-99 | Gobo4shake slow to fast |
|  |  | 100-108 | Gobo5shake slow to fast |
|  |  | 109-117 | Gobo6shake slow to fast |
|  |  | 118-127 | Gobo7shake slow to fast |
|  |  | 128-189 | Rotate forward, going from fast to slow |
|  |  | 190-193 | Stop |
|  |  | 194-255 | Reverse rotation, slow to fast |
| 12 | Gobo Rotation | 0-127 | 0 - 360° |
|  |  | 128-189 | Rotate forward, going from fast to slow |
|  |  | 190-193 | stop |
|  |  | 194-255 | Reverse rotation, from fast to slow |
| 13 | Gobo Rotation fine | 0-255 | 0-100% |
| 14 | Fixed Gobo | 0-7 | Open |
|  |  | 8-13 | Gobo1 |
|  |  | 14-19 | Gobo2 |
|  |  | 20-25 | Gobo3 |
|  |  | 26-31 | Gobo4 |
|  |  | 32-37 | Gobo5 |
|  |  | 38-43 | Gobo6 |
|  |  | 44-49 | Gobo7 |
|  |  | 50-55 | Gobo8 |
|  |  | 56-63 | Gobo9 |
|  |  | 64-70 | Gobo1shake slow to fast |
|  |  | 71-77 | Gobo2shake slow to fast |
|  |  | 78-84 | Gobo3shake slow to fast |
|  |  | 85-91 | Gobo4shake slow to fast |
|  |  | 92-98 | Gobo5shake slow to fast |
|  |  | 99-105 | Gobo6shake slow to fast |
|  |  | 106-112 | Gobo7shake slow to fast |
|  |  | 113-119 | Gobo8shake slow to fast |
|  |  | 120-127 | Gobo9shake slow to fast |
|  |  | 128-189 | Reverse rotation, slow to fast |
|  |  | 190-193 | Stop |
|  |  | 194-255 | Rotate forward, going from fast to slow |
| 15 | Animation Gobo | 0-7 | Open |
|  |  | 8-129 | Reverse rotation, slow to fast |
|  |  | 130-133 | Stop |
|  |  | 134-255 | Rotate forward, going from fast to slow |
| 16 | Iris | 0-255 | 100%-0% |
| 17 | 4 prism | 0-7 | No function |
|  |  | 8-255 | Prism cut |
| 18 | 4 Prism Rotation | 0-127 | 0 - 360° |
|  |  | 128-189 | Reverse rotation, slow to fast |
|  |  | 190-193 | Stop |
|  |  | 194-255 | Rotate forward, going from fast to slow |
| 19 | 4 row prism | 0-7 | No function |
|  |  | 8-255 | Prism cut |
| 20 | 4 row prism Rotation | 0-127 | 0 - 360° |
|  |  | 128-189 | Reverse rotation, slow to fast |
|  |  | 190-193 | Stop |
|  |  | 194-255 | Rotate forward, going from fast to slow |
| 21 | CRI | 0-7 | No function |
|  |  | 8-255 | CRI cut |
| 22 | Frost 1 | 0-255 | 0-100% |
| 23 | Frost 2 | 0-255 | 0-100% |
| 24 | zoom | 0-255 | 100%-0% |
| 25 | Zoom Fine | 0-255 | 100%-0% |
| 26 | focus | 0-255 | 0-100% |
| 27 | Focus Fine | 0-255 | 0-100% |
| 28 | strobe | 0-7 | Close |
|  |  | 8-15 | Open |
|  |  | 16-131 | Strobe slow to fast |
|  |  | 132-139 | Open 1 |
|  |  | 140-181 | Open quickly, close slowly, from slow to fast |
|  |  | 182-189 | Open 2 |
|  |  | 190-231 | Slow opening and fast closing, from slow to fast |
|  |  | 232-239 | Open 3 |
|  |  | 240-247 | Random strobe, from slow to fast |
|  |  | 248-255 | Open 5 |
| 29 | dimmer | 0-255 | Dimming 0% to 100% |
| 30 | Dimmer fine | 0-255 | Dimming fine 0% to 100% |
| 31 | Cut rotation | 0-255 | 0-120° |
| 32 | Cut down 1 | 0-255 | 0-100% |
| 33 | Cut down 2 | 0-255 | 0-100% |
| 34 | Cut on1 | 0-255 | 0-100% |
| 35 | Cut on 2 | 0-255 | 0-100% |
| 36 | Cut left1 | 0-255 | 0-100% |
| 37 | Cut left2 | 0-255 | 0-100% |
| 38 | Cut right1 | 0-255 | 0-100% |
| 39 | Cut right2 | 0-255 | 0-100% |
| 40 | Function settings | 0-29 | No function |
|  |  | 30-39 | Dimming curve: square law |
|  |  | 40-49 | Dimming curve: inverse square law |
|  |  | 50-59 | Dimming curve: Linear dimmer |
|  |  | 60-69 | Dimming curve: S- curve |
|  |  | 70-79 | Fan mode: Auto |
|  |  | 80-89 | Fan mode: Low speed |
|  |  | 90-99 | Fan mode: High speed |
|  |  | 100-119 | No function |
|  |  | 120-129 | 1200Hz |
|  |  | 130-139 | 2400Hz |
|  |  | 140-149 | 4000Hz |
|  |  | 150-159 | 6000Hz |
|  |  | 160-169 | 25000Hz |
|  |  | 170-179 | No function |
|  |  | 180-189 | Dimming speed: fast |
|  |  | 190-199 | Dimming speed: Slow |
|  |  | 200-209 | Reset all motors |
|  |  | 210-219 | Head motor reset |
|  |  | 220-229 | P/T motor reset |
|  |  | 230-255 | No function |

- 39Channels

| CH | Function | Value | Effect |
| --- | --- | --- | --- |
| 1 | Pan | 0-255 | Pan 0-540° |
| 2 | Pan Fine | 0-255 | Pan Fine |
| 3 | Tilt | 0-255 | Tilt 0-270° |
| 4 | Tilt Fine | 0-255 | Tilt Fine |
| 5 | P/T Speed | 0-255 | Fast to slow |
| 6 | C | 0-255 | 0-100% |
| 7 | M | 0-255 | 0-100% |
| 8 | Y | 0-255 | 0-100% |
| 9 | CTO | 0-255 | 0-100% |
| 10 | Color | 0-7 | Open |
|  |  | 8-18 | color1 |
|  |  | 19-29 | color2 |
|  |  | 30-40 | color3 |
|  |  | 41-51 | color4 |
|  |  | 52-63 | color5 |
|  |  | 64-67 | Open 1 |
|  |  | 68-71 | Open+Color 1 |
|  |  | 72-75 | Color 1 |
|  |  | 76-79 | Color 1+Color 2 |
|  |  | 80-83 | Color 2 |
|  |  | 84-87 | Color 2+Color 3 |
|  |  | 88-91 | Color 3 |
|  |  | 92-95 | Color 3+Color 4 |
|  |  | 96-99 | Color 4 |
|  |  | 100-103 | Color 4+Color 5 |
|  |  | 104-107 | Color 5 |
|  |  | 108-111 | Color 5+CRI |
|  |  | 112-115 | CRI |
|  |  | 116-119 | CRI+Open |
|  |  | 120-127 | Open 2 |
|  |  | 128-189 | Reverse rotation, from fast to slow |
|  |  | 190-193 | Stop |
|  |  | 194-255 | Rotate forward, going from fast to slow |
| 11 | Rotation Gobo | 0-7 | Open |
|  |  | 8-15 | Gobo1 |
|  |  | 16-23 | Gobo2 |
|  |  | 24-31 | Gobo3 |
|  |  | 32-39 | Gobo4 |
|  |  | 40-47 | Gobo5 |
|  |  | 48-55 | Gobo6 |
|  |  | 56-63 | Gobo7 |
|  |  | 64-72 | Gobo1shake slow to fast |
|  |  | 73-81 | Gobo2shake slow to fast |
|  |  | 82-90 | Gobo3shake slow to fast |
|  |  | 91-99 | Gobo4shake slow to fast |
|  |  | 100-108 | Gobo5shake slow to fast |
|  |  | 109-117 | Gobo6shake slow to fast |
|  |  | 118-127 | Gobo7shake slow to fast |
|  |  | 128-189 | Rotate forward, going from fast to slow |
|  |  | 190-193 | Stop |
|  |  | 194-255 | Reverse rotation, slow to fast |
| 12 | Gobo Rotation | 0-127 | 0 - 360° |
|  |  | 128-189 | Rotate forward, going from fast to slow |
|  |  | 190-193 | stop |
|  |  | 194-255 | Reverse rotation, from fast to slow |
| 13 | Fixed Gobo | 0-7 | Open |
|  |  | 8-13 | Gobo1 |
|  |  | 14-19 | Gobo2 |
|  |  | 20-25 | Gobo3 |
|  |  | 26-31 | Gobo4 |
|  |  | 32-37 | Gobo5 |
|  |  | 38-43 | Gobo6 |
|  |  | 44-49 | Gobo7 |
|  |  | 50-55 | Gobo8 |
|  |  | 56-63 | Gobo9 |
|  |  | 64-70 | Gobo1shake slow to fast |
|  |  | 71-77 | Gobo2shake slow to fast |
|  |  | 78-84 | Gobo3shake slow to fast |
|  |  | 85-91 | Gobo4shake slow to fast |
|  |  | 92-98 | Gobo5shake slow to fast |
|  |  | 99-105 | Gobo6shake slow to fast |
|  |  | 106-112 | Gobo7shake slow to fast |
|  |  | 113-119 | Gobo8shake slow to fast |
|  |  | 120-127 | Gobo9shake slow to fast |
|  |  | 128-189 | Reverse rotation, slow to fast |
|  |  | 190-193 | Stop |
|  |  | 194-255 | Rotate forward, going from fast to slow |
| 14 | Animation Gobo | 0-7 | Open |
|  |  | 8-129 | Reverse rotation, slow to fast |
|  |  | 130-133 | Stop |
|  |  | 134-255 | Rotate forward, going from fast to slow |
| 15 | Iris | 0-255 | 100%-0% |
| 16 | 4 prism | 0-7 | No function |
|  |  | 8-255 | Prism cut |
| 17 | 4 Prism Rotation | 0-127 | 0 - 360° |
|  |  | 128-189 | Reverse rotation, slow to fast |
|  |  | 190-193 | Stop |
|  |  | 194-255 | Rotate forward, going from fast to slow |
| 18 | 4 row prism | 0-7 | No function |
|  |  | 8-255 | Prism cut |
| 19 | 4 row prism Rotation | 0-127 | 0 - 360° |
|  |  | 128-189 | Reverse rotation, slow to fast |
|  |  | 190-193 | Stop |
|  |  | 194-255 | Rotate forward, going from fast to slow |
| 20 | CRI | 0-7 | No function |
|  |  | 8-255 | CRI cut |
| 21 | Frost 1 | 0-255 | 0-100% |
| 22 | Frost 2 | 0-255 | 0-100% |
| 23 | zoom | 0-255 | 100%-0% |
| 24 | Zoom Fine | 0-255 | 100%-0% |
| 25 | focus | 0-255 | 0-100% |
| 26 | Focus Fine | 0-255 | 0-100% |
| 27 | strobe | 0-7 | Close |
|  |  | 8-15 | Open |
|  |  | 16-131 | Strobe slow to fast |
|  |  | 132-139 | Open 1 |
|  |  | 140-181 | Open quickly, close slowly, from slow to fast |
|  |  | 182-189 | Open 2 |
|  |  | 190-231 | Slow opening and fast closing, from slow to fast |
|  |  | 232-239 | Open 3 |
|  |  | 240-247 | Random strobe, from slow to fast |
|  |  | 248-255 | Open 5 |
| 28 | dimmer | 0-255 | Dimming 0% to 100% |
| 29 | Dimmer fine | 0-255 | Dimming fine 0% to 100% |
| 30 | Cut rotation | 0-255 | 0-120° |
| 31 | Cut down 1 | 0-255 | 0-100% |
| 32 | Cut down 2 | 0-255 | 0-100% |
| 33 | Cut on1 | 0-255 | 0-100% |
| 34 | Cut on 2 | 0-255 | 0-100% |
| 35 | Cut left1 | 0-255 | 0-100% |
| 36 | Cut left2 | 0-255 | 0-100% |
| 37 | Cut right1 | 0-255 | 0-100% |
| 38 | Cut right2 | 0-255 | 0-100% |
| 39 | Function settings | 0-29 | No function |
|  |  | 30-39 | Dimming curve: square law |
|  |  | 40-49 | Dimming curve: inverse square law |
|  |  | 50-59 | Dimming curve: Linear dimmer |
|  |  | 60-69 | Dimming curve: S- curve |
|  |  | 70-79 | Fan mode: Auto |
|  |  | 80-89 | Fan mode: Low speed |
|  |  | 90-99 | Fan mode: High speed |
|  |  | 100-119 | No function |
|  |  | 120-129 | 1200Hz |
|  |  | 130-139 | 2400Hz |
|  |  | 140-149 | 4000Hz |
|  |  | 150-159 | 6000Hz |
|  |  | 160-169 | 25000Hz |
|  |  | 170-179 | No function |
|  |  | 180-189 | Dimming speed: fast |
|  |  | 190-199 | Dimming speed: Slow |
|  |  | 200-209 | Reset all motors |
|  |  | 210-219 | Head motor reset |
|  |  | 220-229 | P/T motor reset |
|  |  | 230-255 | No function |

- 31 Channels

| CH | Function | Value | Effect |
| --- | --- | --- | --- |
| 1 | Pan | 0-255 | Pan 0-540° |
| 2 | Pan Fine | 0-255 | Pan Fine |
| 3 | Tilt | 0-255 | Tilt 0-270° |
| 4 | Tilt Fine | 0-255 | Tilt Fine |
| 5 | P/T Speed | 0-255 | Fast to slow |
| 6 | C | 0-255 | 0-100% |
| 7 | M | 0-255 | 0-100% |
| 8 | Y | 0-255 | 0-100% |
| 9 | CTO | 0-255 | 0-100% |
| 10 | Color | 0-7 | Open |
|  |  | 8-18 | color1 |
|  |  | 19-29 | color2 |
|  |  | 30-40 | color3 |
|  |  | 41-51 | color4 |
|  |  | 52-63 | color5 |
|  |  | 64-67 | Open 1 |
|  |  | 68-71 | Open+Color 1 |
|  |  | 72-75 | Color 1 |
|  |  | 76-79 | Color 1+Color 2 |
|  |  | 80-83 | Color 2 |
|  |  | 84-87 | Color 2+Color 3 |
|  |  | 88-91 | Color 3 |
|  |  | 92-95 | Color 3+Color 4 |
|  |  | 96-99 | Color 4 |
|  |  | 100-103 | Color 4+Color 5 |
|  |  | 104-107 | Color 5 |
|  |  | 108-111 | Color 5+CRI |
|  |  | 112-115 | CRI |
|  |  | 116-119 | CRI+Open |
|  |  | 120-127 | Open 2 |
|  |  | 128-189 | Reverse rotation, from fast to slow |
|  |  | 190-193 | Stop |
|  |  | 194-255 | Rotate forward, going from fast to slow |
| 11 | Iris | 0-255 | 100%-0% |
| 12 | CRI | 0-7 | No function |
|  |  | 8-255 | CRI cut |
| 13 | Frost 1 | 0-255 | 0-100% |
| 14 | Frost 2 | 0-255 | 0-100% |
| 15 | zoom | 0-255 | 100%-0% |
| 16 | Zoom Fine | 0-255 | 100%-0% |
| 17 | focus | 0-255 | 0-100% |
| 18 | Focus Fine | 0-255 | 0-100% |
| 19 | strobe | 0-7 | Close |
|  |  | 8-15 | Open |
|  |  | 16-131 | Strobe slow to fast |
|  |  | 132-139 | Open 1 |
|  |  | 140-181 | Open quickly, close slowly, from slow to fast |
|  |  | 182-189 | Open 2 |
|  |  | 190-231 | Slow opening and fast closing, from slow to fast |
|  |  | 232-239 | Open 3 |
|  |  | 240-247 | Random strobe, from slow to fast |
|  |  | 248-255 | Open 5 |
| 20 | dimmer | 0-255 | Dimming 0% to 100% |
| 21 | Dimmer fine | 0-255 | Dimming fine 0% to 100% |
| 22 | Cut rotation | 0-255 | 0-120° |
| 23 | Cut down 1 | 0-255 | 0-100% |
| 24 | Cut down 2 | 0-255 | 0-100% |
| 25 | Cut on1 | 0-255 | 0-100% |
| 26 | Cut on 2 | 0-255 | 0-100% |
| 27 | Cut left1 | 0-255 | 0-100% |
| 28 | Cut left2 | 0-255 | 0-100% |
| 29 | Cut right1 | 0-255 | 0-100% |
| 30 | Cut right2 | 0-255 | 0-100% |
| 31 | Function settings | 0-29 | No function |
|  |  | 30-39 | Dimming curve: square law |
|  |  | 40-49 | Dimming curve: inverse square law |
|  |  | 50-59 | Dimming curve: Linear dimmer |
|  |  | 60-69 | Dimming curve: S- curve |
|  |  | 70-79 | Fan mode: Auto |
|  |  | 80-89 | Fan mode: Low speed |
|  |  | 90-99 | Fan mode: High speed |
|  |  | 100-119 | No function |
|  |  | 120-129 | 1200Hz |
|  |  | 130-139 | 2400Hz |
|  |  | 140-149 | 4000Hz |
|  |  | 150-159 | 6000Hz |
|  |  | 160-169 | 25000Hz |
|  |  | 170-179 | No function |
|  |  | 180-189 | Dimming speed: fast |
|  |  | 190-199 | Dimming speed: Slow |
|  |  | 200-209 | Reset all motors |
|  |  | 210-219 | Head motor reset |
|  |  | 220-229 | P/T motor reset |
|  |  | 230-255 | No function |

- 30Channels

| CH | Function | Value | Effect |
| --- | --- | --- | --- |
| 1 | Pan | 0-255 | Pan 0-540° |
| 2 | Pan Fine | 0-255 | Pan Fine |
| 3 | Tilt | 0-255 | Tilt 0-270° |
| 4 | Tilt Fine | 0-255 | Tilt Fine |
| 5 | P/T Speed | 0-255 | Fast to slow |
| 6 | C | 0-255 | 0-100% |
| 7 | M | 0-255 | 0-100% |
| 8 | Y | 0-255 | 0-100% |
| 9 | CTO | 0-255 | 0-100% |
| 10 | Color | 0-7 | Open |
|  |  | 8-18 | color1 |
|  |  | 19-29 | color2 |
|  |  | 30-40 | color3 |
|  |  | 41-51 | color4 |
|  |  | 52-63 | color5 |
|  |  | 64-67 | Open 1 |
|  |  | 68-71 | Open+Color 1 |
|  |  | 72-75 | Color 1 |
|  |  | 76-79 | Color 1+Color 2 |
|  |  | 80-83 | Color 2 |
|  |  | 84-87 | Color 2+Color 3 |
|  |  | 88-91 | Color 3 |
|  |  | 92-95 | Color 3+Color 4 |
|  |  | 96-99 | Color 4 |
|  |  | 100-103 | Color 4+Color 5 |
|  |  | 104-107 | Color 5 |
|  |  | 108-111 | Color 5+CRI |
|  |  | 112-115 | CRI |
|  |  | 116-119 | CRI+Open |
|  |  | 120-127 | Open 2 |
|  |  | 128-189 | Reverse rotation, speed from fast to slow |
|  |  | 190-193 | Stop |
|  |  | 194-255 | Rotate forward, going from fast to slow |
| 11 | Rotation Gobo | 0-7 | Open |
|  |  | 8-15 | Gobo1 |
|  |  | 16-23 | Gobo2 |
|  |  | 24-31 | Gobo3 |
|  |  | 32-39 | Gobo4 |
|  |  | 40-47 | Gobo5 |
|  |  | 48-55 | Gobo6 |
|  |  | 56-63 | Gobo7 |
|  |  | 64-72 | Gobo1shake slow to fast |
|  |  | 73-81 | Gobo2shake slow to fast |
|  |  | 82-90 | Gobo3shake slow to fast |
|  |  | 91-99 | Gobo4shake slow to fast |
|  |  | 100-108 | Gobo5shake slow to fast |
|  |  | 109-117 | Gobo6shake slow to fast |
|  |  | 118-127 | Gobo7shake slow to fast |
|  |  | 128-189 | Rotate forward, going from fast to slow |
|  |  | 190-193 | Stop |
|  |  | 194-255 | Reverse rotation, slow to fast |
| 12 | Gobo Rotation | 0-127 | 0 - 360° |
|  |  | 128-189 | Rotate forward, going from fast to slow |
|  |  | 190-193 | stop |
|  |  | 194-255 | Reverse rotation, from fast to slow |
| 13 | Fixed Gobo | 0-7 | Open |
|  |  | 8-13 | Gobo1 |
|  |  | 14-19 | Gobo2 |
|  |  | 20-25 | Gobo3 |
|  |  | 26-31 | Gobo4 |
|  |  | 32-37 | Gobo5 |
|  |  | 38-43 | Gobo6 |
|  |  | 44-49 | Gobo7 |
|  |  | 50-55 | Gobo8 |
|  |  | 56-63 | Gobo9 |
|  |  | 64-70 | Gobo1shake slow to fast |
|  |  | 71-77 | Gobo2shake slow to fast |
|  |  | 78-84 | Gobo3shake slow to fast |
|  |  | 85-91 | Gobo4shake slow to fast |
|  |  | 92-98 | Gobo5shake slow to fast |
|  |  | 99-105 | Gobo6shake slow to fast |
|  |  | 106-112 | Gobo7shake slow to fast |
|  |  | 113-119 | Gobo8shake slow to fast |
|  |  | 120-127 | Gobo9shake slow to fast |
|  |  | 128-189 | Reverse rotation, slow to fast |
|  |  | 190-193 | Stop |
|  |  | 194-255 | Rotate forward, going from fast to slow |
| 14 | Animation Gobo | 0-7 | Open |
|  |  | 8-129 | Reverse rotation, slow to fast |
|  |  | 130-133 | Stop |
|  |  | 134-255 | Rotate forward, going from fast to slow |
| 15 | Iris | 0-255 | 100%-0% |
| 16 | 4 prism | 0-7 | No function |
|  |  | 8-255 | Prism cut |
| 17 | 4 Prism Rotation | 0-127 | 0 - 360° |
|  |  | 128-189 | Reverse rotation, slow to fast |
|  |  | 190-193 | Stop |
|  |  | 194-255 | Rotate forward, going from fast to slow |
| 18 | 4 row prism | 0-7 | No function |
|  |  | 8-255 | Prism cut |
| 19 | 4 row prism Rotation | 0-127 | 0 - 360° |
|  |  | 128-189 | Reverse rotation, slow to fast |
|  |  | 190-193 | Stop |
|  |  | 194-255 | Rotate forward, going from fast to slow |
| 20 | CRI | 0-7 | No function |
|  |  | 8-255 | CRI cut |
| 21 | Frost 1 | 0-255 | 0-100% |
| 22 | Frost 2 | 0-255 | 0-100% |
| 23 | zoom | 0-255 | 100%-0% |
| 24 | Zoom Fine | 0-255 | 100%-0% |
| 25 | focus | 0-255 | 0-100% |
| 26 | Focus Fine | 0-255 | 0-100% |
| 27 | strobe | 0-7 | Close |
|  |  | 8-15 | Open |
|  |  | 16-131 | Strobe slow to fast |
|  |  | 132-139 | Open 1 |
|  |  | 140-181 | Open quickly, close slowly, from slow to fast |
|  |  | 182-189 | Open 2 |
|  |  | 190-231 | Slow opening and fast closing, from slow to fast |
|  |  | 232-239 | Open 3 |
|  |  | 240-247 | Random strobe, from slow to fast |
|  |  | 248-255 | Open 5 |
| 28 | dimmer | 0-255 | Dimming 0% to 100% |
| 29 | Dimmer fine | 0-255 | Dimming fine 0% to 100% |
| 30 | Function settings | 0-29 | No function |
|  |  | 30-39 | Dimming curve: square law |
|  |  | 40-49 | Dimming curve: inverse square law |
|  |  | 50-59 | Dimming curve: Linear dimmer |
|  |  | 60-69 | Dimming curve: S- curve |
|  |  | 70-79 | Fan mode: Auto |
|  |  | 80-89 | Fan mode: Low speed |
|  |  | 90-99 | Fan mode: High speed |
|  |  | 100-119 | No function |
|  |  | 120-129 | 1200Hz |
|  |  | 130-139 | 2400Hz |
|  |  | 140-149 | 4000Hz |
|  |  | 150-159 | 6000Hz |
|  |  | 160-169 | 25000Hz |
|  |  | 170-179 | No function |
|  |  | 180-189 | Dimming speed: fast |
|  |  | 190-199 | Dimming speed: Slow |
|  |  | 200-209 | Reset all motors |
|  |  | 210-219 | Head motor reset |
|  |  | 220-229 | P/T motor reset |
|  |  | 230-255 | No function |

- 22Channels

| CH | Function | Value | Effect |
| --- | --- | --- | --- |
| 1 | Pan | 0-255 | Pan 0-540° |
| 2 | Pan Fine | 0-255 | Pan Fine |
| 3 | Tilt | 0-255 | Tilt 0-270° |
| 4 | Tilt Fine | 0-255 | Tilt Fine |
| 5 | P/T Speed | 0-255 | Fast to slow |
| 6 | C | 0-255 | 0-100% |
| 7 | M | 0-255 | 0-100% |
| 8 | Y | 0-255 | 0-100% |
| 9 | CTO | 0-255 | 0-100% |
| 10 | Color | 0-7 | Open |
|  |  | 8-18 | color1 |
|  |  | 19-29 | color2 |
|  |  | 30-40 | color3 |
|  |  | 41-51 | color4 |
|  |  | 52-63 | color5 |
|  |  | 64-67 | Open 1 |
|  |  | 68-71 | Open+Color 1 |
|  |  | 72-75 | Color 1 |
|  |  | 76-79 | Color 1+Color 2 |
|  |  | 80-83 | Color 2 |
|  |  | 84-87 | Color 2+Color 3 |
|  |  | 88-91 | Color 3 |
|  |  | 92-95 | Color 3+Color 4 |
|  |  | 96-99 | Color 4 |
|  |  | 100-103 | Color 4+Color 5 |
|  |  | 104-107 | Color 5 |
|  |  | 108-111 | Color 5+CRI |
|  |  | 112-115 | CRI |
|  |  | 116-119 | CRI+Open |
|  |  | 120-127 | Open 2 |
|  |  | 128-189 | Reverse rotation,from fast to slow |
|  |  | 190-193 | Stop |
|  |  | 194-255 | Rotate forward, going from fast to slow |
| 11 | Iris | 0-255 | 100%-0% |
| 12 | CRI | 0-7 | No function |
|  |  | 8-255 | CRI cut |
| 13 | Frost 1 | 0-255 | 0-100% |
| 14 | Frost 2 | 0-255 | 0-100% |
| 15 | zoom | 0-255 | 100%-0% |
| 16 | Zoom Fine | 0-255 | 100%-0% |
| 17 | focus | 0-255 | 0-100% |
| 18 | Focus Fine | 0-255 | 0-100% |
| 19 | strobe | 0-7 | Close |
|  |  | 8-15 | Open |
|  |  | 16-131 | Strobe slow to fast |
|  |  | 132-139 | Open 1 |
|  |  | 140-181 | Open quickly, close slowly, from slow to fast |
|  |  | 182-189 | Open 2 |
|  |  | 190-231 | Slow opening and fast closing, from slow to fast |
|  |  | 232-239 | Open 3 |
|  |  | 240-247 | Random strobe, from slow to fast |
|  |  | 248-255 | Open 5 |
| 20 | dimmer | 0-255 | Dimming 0% to 100% |
| 21 | Dimmer fine | 0-255 | Dimming fine 0% to 100% |
| 22 | Function settings | 0-29 | No function |
|  |  | 30-39 | Dimming curve: square law |
|  |  | 40-49 | Dimming curve: inverse square law |
|  |  | 50-59 | Dimming curve: Linear dimmer |
|  |  | 60-69 | Dimming curve: S- curve |
|  |  | 70-79 | Fan mode: Auto |
|  |  | 80-89 | Fan mode: Low speed |
|  |  | 90-99 | Fan mode: High speed |
|  |  | 100-119 | No function |
|  |  | 120-129 | 1200Hz |
|  |  | 130-139 | 2400Hz |
|  |  | 140-149 | 4000Hz |
|  |  | 150-159 | 6000Hz |
|  |  | 160-169 | 25000Hz |
|  |  | 170-179 | No function |
|  |  | 180-189 | Dimming speed: fast |
|  |  | 190-199 | Dimming speed: Slow |
|  |  | 200-209 | Reset all motors |
|  |  | 210-219 | Head motor reset |
|  |  | 220-229 | P/T motor reset |
|  |  | 230-255 | No function |

11.Troubleshooting

Following are a few common problems that may occur during operation. Here are some

suggestions for troubleshooting:

A. The unit does not work, no light and the fan does not work

1. Check the connected power and main fuse.

2. Measure the voltage.

3. Check the power indicator to see whether it can be lit up or not.

B. Not responding to the DMX controller

1. Check whether the DMX connectors and the DMX cables are connected correctly.

2. Check whether the DMX address is correctly set.

3. If the intermittent DMX signal problem occurs, check whether the XLR socket and the signal

cable are well connected.

4. Try it with another DMX controller.

5. Check whether the DMX cables run near or alongside to the high-voltage cables, which may

damage or interfere with the signal circuit.

C. One of the channels is not working well

1. The stepper motor might be damaged or the cable connected to the PCB might be broken.

2. The motor’s drive IC on the PCB might be out of condition.

12.Fixture cleaning

It is absolutely essential that the fixture is kept clean to ensure the maximum light-output and allow

the fixture to function reliably throughout its life. The fixture must be cleaned regularly to avoid dust,

dirt and smoke-fluid residues building up on or within the fixture. The cleaning frequency depends on

the application environment. Clean the fixture immediately if the dust enters it to avoid damage to

the optical lens due to excessive dust.

 A soft lint-free cloth moistened with any good glass cleaning fluid is recommended, under no

circumstances should solvents be used.

 Always dry the parts carefully.

 Clean the external optical lens at least every 20 days and the internal optical lens every 30 days.